## SIMATS ENGINEERING

### ASSESSMENT TOOL 3

#### COURSE NAME: Computer Networks COURSE CODE: CSA0706

COURSE OUTCOME COVERED

CO4: Design network topologies — star, mesh, bus, and hybrid — using networking devices, transmission media, and cabling standards. (BL2, BL6)

Assessment Tool Weightage: 30%

Questions: 5	Total Marks: 25

Annexure A

#### Comparative Analysis

Code of Conduct:

I S. Velarasan / 192571087 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

S. Velarasan

# CSA07 — COMPUTER NETWORKS

### CO4 Assessment Tool 3 — Comparative Analysis: Answers

CO4: Design network topologies — star, mesh, bus, and hybrid

##### Q1. Star vs. Bus Topology — Cost, Reliability, and Performance

Star and bus topologies provide different trade-offs in cost, reliability, and performance. Star normally needs more cabling and equipment, while bus is simpler and cheaper but less dependable.

Comparison

| Criterion | Star Topology | Bus Topology |
| --- | --- | --- |
| Cost | Higher — requires a central switch/hub and a separate cable to each device | Lower — uses one shared backbone with simple connections |
| Reliability | High — failure of one device or its cable normally affects only that device | Low — a backbone failure can interrupt the complete segment |
| Performance | Consistent — switched dedicated links reduce collisions | Falls as more devices share the same transmission medium |

Why the Difference Exists

In a star network, every device has a separate connection to a central switch, which supports efficient full-duplex communication. In a bus network, all devices share one backbone. As more devices transmit, collisions and retransmissions can increase, so performance becomes less consistent.

Real-World Example

Older Ethernet standards such as 10BASE2 and 10BASE5 used shared coaxial bus wiring. It was economical, but a damaged connector or backbone could interrupt the segment. Modern office LANs generally use switched star wiring because its improved reliability and performance justify the extra hardware and cabling.

##### Q2. Mesh vs. Star Topology — Fault Tolerance

Mesh and star show a major difference in fault tolerance because their connections are arranged differently. Mesh has alternate routes, while star depends on its central connecting device.

Comparison

| Criterion | Mesh Topology | Star Topology |
| --- | --- | --- |
| Path redundancy | Several independent routes can connect two devices | Communication normally follows the route through the central device |

| Criterion | Mesh Topology | Star Topology |
| --- | --- | --- |
| Effect of a link/node failure | Traffic can move through another available route | Only the device using the failed link is disconnected |
| Effect of a central-point failure | There is no single central device on which the whole mesh depends | Failure of the central switch/hub can stop the whole network |
| Overall fault tolerance | Very high | Moderate — depends strongly on the central device |

Why the Difference Exists

A full mesh with n nodes contains n(n−1)/2 links, providing multiple possible routes. If one connection fails, traffic can often use another route. In star topology, all devices depend on the central switch or hub, making that device a single important failure point.

Real-World Example

Backbone and ISP networks commonly use partial mesh structures so traffic can be redirected after a link or router failure. In a typical office star network, failure of the central switch can disconnect all attached users.

Q3. Bus vs. Mesh Topology — Installation Complexity

Q3. Bus vs. Mesh Topology — Installation Complexity

Comparison

| Criterion | Bus Topology | Mesh Topology |
| --- | --- | --- |
| Cabling required | One shared backbone plus a drop connection for each device | Each node connects to every other node — n(n−1)/2 links in a full mesh |
| Interfaces needed per node | 1 | (n − 1) |
| How install effort scales | Approximately linear as devices are added | Increases quadratically as devices are added |
| Practical scale | Suitable for small, low-cost networks | Best limited to a small number of critical nodes |

Why the Difference Exists — Worked Example

For 5 nodes, a bus needs one backbone and five drop connections, while a full mesh needs 5×4÷2 = 10 links. With 10 nodes, the bus still has one backbone and ten drops, but a full mesh requires 10×9÷2 = 45 links.

The number of mesh links increases rapidly as nodes are added, and each node needs more interfaces. Therefore, full mesh becomes expensive and difficult to install for ordinary end-user devices.

Real-World Example

Mesh cabling is normally chosen for a small group of important devices, such as core routers, where redundancy is valuable. Bus topology was historically useful for small networks because it required less cabling and simpler installation.

Q4. Hybrid vs. Star Topology — Scalability

Q4. Hybrid vs. Star Topology — Scalability

Comparison

| Criterion | Hybrid Topology | Star Topology |
| --- | --- | --- |
| Basic structure | Combines two or more topologies, such as multiple star sections connected by a backbone | All devices connect directly to one central device |
| How it expands | Add another star section or extend the backbone | Increase central switch capacity or add another switch |
| Growth ceiling | High — mainly constrained by backbone capacity | Constrained by the ports/capacity of the central device |
| Typical scale of use | Large campuses, enterprises, and multi-building networks | Single offices, floors, or small LANs |

Why the Difference Exists

A single star network is limited by the capacity and ports of its central switch. When additional sections are connected through more switches and a backbone, the design becomes hybrid. Hybrid topology therefore makes network expansion more systematic.

Real-World Example

A university or corporate campus can use a hybrid structure in which each building or department has its own star LAN. Local switches then connect to a larger distribution or core network, making it easier to add new sections.

Q5. Maintenance Comparison — Star, Mesh, Bus, and Hybrid Topologies

Q5. Maintenance Comparison — Star, Mesh, Bus, and Hybrid Topologies

Comparison

| Topology | Fault Isolation | Troubleshooting Effort | Overall Maintenance |
| --- | --- | --- | --- |
| Star | Easy — switch-port information helps locate the affected device or cable | Low | Easy |
| Mesh | Difficult — many alternative links need to be checked | High | Difficult |
| Bus | Difficult — finding a break in the shared backbone can take physical testing | High | Difficult |
| Hybrid | Depends on the section; local stars are easier than the backbone | Medium | Moderate |

Why the Difference Exists

Star is usually easiest to maintain because switch-port information helps identify a failed device or cable. Bus is harder because a backbone fault may require physical testing. Mesh has many links to monitor, while hybrid combines simple local star sections with a more complex backbone.

Real-World Example

Network teams can use monitoring dashboards effectively for star and hybrid access networks because individual ports provide status information. A meshed core needs more detailed topology and path monitoring because it contains many possible links and failure points.