SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE: CSA0713     COURSE NAME: COMPUTER NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I (Velarasan .S/192571087)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Velarasan.S

1. Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

Answer:

Step 1 — Identify the given values:

• Message M = 11100011 (8 bits)

• Generator polynomial G = X3 + 1 → in binary, G = 1001 (4 bits)

• Degree of G, r = 3 (highest power of X)

Step 2 — Append r zero bits to the message:

Since r = 3, append 3 zeros to M before division:

M · X3  =  11100011  +  000  =  11100011000  (11 bits)

Step 3 — Perform mod-2 (XOR) long division by G = 1001:

Divide 11100011000 by 1001, XOR-ing with the divisor whenever the leading bit of the working remainder is 1, and bringing down one bit at a time (7 division steps in total since 11 − 4 + 1 = 8 quotient bits are produced):

1)  1110 ÷ 1001 → XOR → 0111

2)  1110 ÷ 1001 → XOR → 0111  (bring down next 0)

3)  1111 ÷ 1001 → XOR → 0110  (bring down next 1)

4)  1101 ÷ 1001 → XOR → 0100  (bring down next 1)

5)  1000 ÷ 1001 → XOR → 0001  (bring down next 0)

6)  0010 → leading bit 0, no XOR  (bring down next 0)

7)  Remaining 3 bits after all bits consumed → remainder

Remainder (CRC) = 100

Step 4 — Form the transmitted codeword T:

Replace the 3 appended zero bits with the computed remainder (i.e., T = M·X3 XOR Remainder):

T = 11100011  +  100  =  11100011100

Step 5 — Verify the codeword:

Dividing T = 11100011100 by G = 1001 gives a remainder of 000, confirming T is exactly divisible by the generator polynomial — this is the required property of a valid CRC codeword.

→ Transmitted message T = 11100011100

Part (ii) — Effect of an inverted leftmost bit

Step 1 — Apply the error:

Flipping the leftmost bit of T (1 → 0) changes it from:

T  = 11100011100

R' = 01100011100  (received, with error)

Step 2 — Divide the received word R' by G = 1001:

Because CRC division is linear under XOR, the result equals (T mod G) XOR (E mod G), where E is the error pattern (a single 1 at the leftmost/position-10 bit, i.e., E corresponds to X10). Since T mod G = 0, the receiver's remainder is simply E mod G.

• In this arithmetic, X3 ≡ 1 (mod G), so X10 = (X3)3 · X ≡ 13 · X = X

• So E mod G = X, which as a 3-bit remainder is written as 010.

Step 3 — Result of receiver's calculation:

Remainder obtained by receiver = 010 (non-zero)

Step 4 — How the receiver detects the error:

• A valid, error-free codeword must always give a remainder of exactly 000 when divided by G.

• Here the receiver's division produces 010, not 000.

• Since the remainder is non-zero, the receiver immediately knows the received bit pattern is not a valid codeword, and therefore concludes that a transmission error has occurred — without needing to know the correct message.

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

Answer:

Step 1 — Given values:

• Bandwidth = 10 Mbps (this is the network capacity, not directly needed for the throughput calculation itself)

• Frames per minute = 12,000

• Bits per frame = 10,000

Step 2 — Convert frames per minute to frames per second:

Frames per second = 12,000 / 60 = 200 frames/sec

Step 3 — Apply the throughput formula:

Throughput = (frames per second) × (bits per frame)

Throughput = 200 × 10,000 = 2,000,000 bps

Step 4 — Convert to Mbps and interpret:

Throughput = 2 Mbps

Even though the network's bandwidth is 10 Mbps, the actual achieved throughput is only 2 Mbps, because throughput measures the actual data successfully delivered per second, not the theoretical maximum capacity of the link.

2. b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal-to-noise ratio is 3162. Find the channel capacity.

Answer:

Step 1 — Given values:

• Bandwidth, B = 3000 Hz

• Signal-to-Noise Ratio, SNR = 3162

Step 2 — Write Shannon's channel capacity formula:

C = B × log2(1 + SNR)

Step 3 — Substitute the given values:

C = 3000 × log2(1 + 3162) = 3000 × log2(3163)

Step 4 — Evaluate the logarithm:

log2(3163) ≈ 11.63  (since 211 = 2048 and 212 = 4096, so log2(3163) lies between 11 and 12, closer to 11.6)

Step 5 — Compute the final capacity:

C ≈ 3000 × 11.63 ≈ 34,860 bps

→ Channel capacity ≈ 34.86 kbps (≈ 34,860 bps)

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

Answer:

Step 1 — Structure of the code:

• Hamming(12,8) encodes 8 data bits into a 12-bit codeword.

• Number of redundant parity bits added = 12 − 8 = 4, denoted r1, r2, r4, r8.

• These parity bits are placed at bit positions that are powers of 2: positions 1, 2, 4, and 8. The 8 data bits occupy the remaining positions (3, 5, 6, 7, 9, 10, 11, 12).

Step 2 — How each parity bit is calculated:

• Every bit position from 1 to 12 is written in binary.

• A parity bit at position 2k covers (is calculated over) every position whose binary representation has a 1 in the k-th bit place.

• For example, r1 (position 1) checks all positions with a 1 in the least-significant bit (1,3,5,7,9,11); r2 (position 2) checks positions 2,3,6,7,10,11; and so on for r4 and r8.

• Each parity bit is set (even parity) so that the total number of 1s in its covered group is even.

Step 3 — Error detection and correction at the receiver:

• The receiver recalculates each of the 4 parity checks on the received 12-bit word.

• The results of these 4 checks are combined (in order r8 r4 r2 r1) to form a 4-bit binary number called the syndrome.

• If the syndrome = 0000 → no error is present.

• If the syndrome is non-zero, its decimal value directly points to the exact bit position (1–12) that is in error.

• The receiver simply inverts (flips) the bit at that identified position — this single-bit self-correction is what makes the code Single-Error-Correcting (SEC).

Step 4 — Relevance to the hospital scenario:

• Since Hamming(12,8) can correct a single-bit error entirely on its own, the central monitoring station recovers the correct heart-rate / blood-pressure reading immediately.

• No retransmission request needs to be sent back to the wearable device.

• This matters because in real-time healthcare monitoring, waiting for a retransmission (as required by detection-only schemes like plain CRC or parity) could delay life-critical data and put the patient at risk — the FEC approach trades a little extra bandwidth (4 redundant bits) for uninterrupted, real-time accuracy.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments.

Answer:

Step 1 — Given values:

• Original network: 172.16.10.0/24

• New subnet mask: /27 → 255.255.255.224

Step 2 — Find the number of subnets:

Bits borrowed from the host portion = new prefix − old prefix = 27 − 24 = 3 bits

Number of subnets = 23 = 8 subnets

Step 3 — Find the usable hosts per subnet:

Host bits remaining = 32 − 27 = 5 bits

Total addresses per subnet = 25 = 32

Subtract 2 reserved addresses (network address and broadcast address):

Usable hosts per subnet = 32 − 2 = 30 usable hosts

Step 4 — Find the block size (subnet increment):

Block size = 256 − 224 (last octet value of the /27 mask) = 32

So each subnet's range spans 32 addresses, giving the 8 subnets:

172.16.10.0, .32, .64, .96, .128, .160, .192, .224  (each a /27 block)

Step 5 — Locate 172.16.10.78 within these subnets:

• The subnet boundaries around 78 are .64 and .96 (since 64 ≤ 78 < 96).

• Subnet: 172.16.10.64/27

• Network address (first address of block): 172.16.10.64

• Broadcast address (last address of block): 172.16.10.95

• Usable host range: 172.16.10.65 – 172.16.10.94

Step 6 — Check whether 172.16.10.95 is in the same subnet:

• The subnet 172.16.10.64/27 covers addresses .64 through .95.

• 95 falls within this range, so yes, 172.16.10.95 belongs to the same subnet.

• However, .95 is specifically the broadcast address of this subnet, so it cannot be assigned to a host device — it is reserved for broadcasting to all devices within the subnet.

4. b) A network uses the IPv6 address 2001:0db8:0000:0000:0000:ff00:0042:8329.

Answer:

Step 1 — Start with the full given address:

2001:0db8:0000:0000:0000:ff00:0042:8329  (8 groups of 16 bits = 128 bits total)

Step 2 — Compress the address (shortest form):

• Rule A: remove leading zeros within each group → 0db8 becomes db8, 0000 becomes 0, 0042 becomes 42.

• Rule B: replace the single longest run of consecutive all-zero groups with '::' (this can be done only once in an address).

• Here, groups 3, 4, and 5 (0000:0000:0000) are all zero, so they are compressed to '::'.

→ Compressed address = 2001:db8::ff00:42:8329

Step 3 — Expand the compressed address back to full notation:

• Reverse the process: '::' is expanded back to the correct number of all-zero groups needed to make 8 groups total, and each group is padded back to 4 hex digits.

→ Expanded address = 2001:0db8:0000:0000:0000:ff00:0042:8329

Step 4 — Determine the network prefix using /64:

• A /64 prefix means the first 64 bits (the first 4 groups of the 8) form the network portion.

• First 4 groups: 2001 : 0db8 : 0000 : 0000

→ Network prefix = 2001:0db8:0000:0000/64  (compressed: 2001:db8::/64)

Step 5 — Calculate the total number of possible host addresses:

• Total address length = 128 bits; Network portion = 64 bits

• Remaining bits available for hosts = 128 − 64 = 64 bits

→ Number of possible host addresses = 264 ≈ 1.844 × 1019 addresses

5. A university is conducting an online semester examination for 8,000 students simultaneously... Analyze the scenario and identify the primary networking problem.

Answer:

Step 1 — Identify the key facts from the scenario:

• Server connection speed: 10 Gbps (very high).

• Student connection speeds: fiber (100 Mbps), Wi-Fi (20 Mbps), mobile (5–10 Mbps) — all much slower than the server.

• Symptoms observed: incomplete downloads, repeated retransmissions, delayed acknowledgments, buffer overflow, temporary freezing.

Step 2 — Identify the primary networking problem:

→ This is a Flow Control mismatch between sender and receiver, which in turn triggers network-level congestion.

The server transmits at a rate suited to its own 10 Gbps link, without adjusting to the much lower reception/processing capacity of each individual student device.

Step 3 — Trace how the mismatch causes each symptom, step by step:

1. The server sends data faster than a slow-link student device (e.g., mobile at 5 Mbps) can receive and process it.

2. The receiver's buffer fills up faster than the application can read data out of it → buffer overflow.

3. Once the buffer is full, any further incoming packets are simply dropped.

4. Because packets are dropped, the corresponding acknowledgments (ACKs) never reach the sender in time.

5. The sender's timers expire without receiving the expected ACKs, so it retransmits those packets — seen as 'repeated retransmissions' and 'delayed acknowledgments' in the scenario.

6. These retransmissions add extra, unnecessary traffic onto a network that is already under load from 8,000 simultaneous students, pushing the network toward congestion.

7. The combined effect (dropped packets + retransmissions + congestion) results in the observed slow downloads and temporary freezing of the exam portal.

Step 4 — How this could be resolved:

• A flow-control mechanism such as the sliding window protocol should be used, where the receiver continuously advertises its available buffer space (receive window) to the sender.

• The sender then transmits only as much data as the slowest/most constrained receiver can currently handle, rather than blasting at its own maximum link speed.

• This prevents buffer overflow, avoids unnecessary retransmissions, and reduces the extra congestion load on the network.