SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0706

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Code of Conduct:

I(S Velarasan/192571087)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

S Velarasan

ANSWERS

Each scenario below is solved step by step, showing the network/broadcast addresses, valid host ranges, usable host counts, and any remaining/unused address space, followed by a brief justification.

# Question 1: Smart Campus Subnetting — 192.168.100.0/24

Given block: 192.168.100.0/24 (256 total addresses). Subnets are carved in order: /25 (Computing), /26 (Electronics), /27 (Mechanical).

## 1 & 2. Network Address, Broadcast Address, and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| School of Computing | /25 (255.255.255.128) | 192.168.100.0 | 192.168.100.127 | 192.168.100.1 – 192.168.100.126 |
| School of Electronics | /26 (255.255.255.192) | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 – 192.168.100.190 |
| School of Mechanical Eng. | /27 (255.255.255.224) | 192.168.100.192 | 192.168.100.223 | 192.168.100.193 – 192.168.100.222 |

## 3. Usable Host Addresses per Subnet

- School of Computing (/25): 2^7 − 2 = 126 usable hosts

- School of Electronics (/26): 2^6 − 2 = 62 usable hosts

- School of Mechanical Engineering (/27): 2^5 − 2 = 30 usable hosts

## 4. Remaining Unused IP Address Range

After allocating .0–.127 (Computing), .128–.191 (Electronics), and .192–.223 (Mechanical), the addresses 192.168.100.224 – 192.168.100.255 (32 addresses, a /27-sized block) remain unused and available for future departments.

## Analysis

Allocating largest-to-smallest (/25 → /26 → /27) keeps each subnet block-aligned with no wasted gaps, and leaves exactly one contiguous /27 free for future expansion — the most efficient use of the /24 block.

# Question 2: Multinational Software Company — 10.10.0.0/16

Given block: 10.10.0.0/16 (65,536 total addresses). Subnets are carved in order: /24 (Software Dev), /25 (QA), /26 (HR), /27 (Executive).

## 1 & 2. Network Address, Broadcast Address, and Valid Host Range

| Department | Subnet Mask | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- | --- |
| Software Development | /24 (255.255.255.0) | 10.10.0.0 | 10.10.0.255 | 10.10.0.1 – 10.10.0.254 |
| Quality Assurance | /25 (255.255.255.128) | 10.10.1.0 | 10.10.1.127 | 10.10.1.1 – 10.10.1.126 |
| Human Resources | /26 (255.255.255.192) | 10.10.1.128 | 10.10.1.191 | 10.10.1.129 – 10.10.1.190 |
| Executive Management | /27 (255.255.255.224) | 10.10.1.192 | 10.10.1.223 | 10.10.1.193 – 10.10.1.222 |

## 3. Usable Host Addresses per Subnet

- Software Development (/24): 2^8 − 2 = 254 usable hosts

- Quality Assurance (/25): 2^7 − 2 = 126 usable hosts

- Human Resources (/26): 2^6 − 2 = 62 usable hosts

- Executive Management (/27): 2^5 − 2 = 30 usable hosts

## 4. Remaining Unused IP Address Range

- Within 10.10.1.0/24: addresses 10.10.1.224 – 10.10.1.255 (32 addresses) remain unused.

- Beyond that: the entire range 10.10.2.0 – 10.10.255.255 (254 more /24-sized blocks) is still completely unallocated within the /16 block.

## Analysis

Only the first two /24 blocks (10.10.0.0/24 and 10.10.1.0/24) of the /16 have been touched; the vast majority of the address space is reserved for future departments, giving the company significant room to grow without re-numbering existing subnets.

# Question 3: Smart Hospital IPv4-to-IPv6 Migration

Given address: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

## 1. Compressed Notation

2001:db8:abcd::/64

Leading zeros in each hextet are dropped (0db8 → db8), and the longest run of consecutive all-zero hextets (the last four groups) is replaced once with '::'.

## 2. Expanded (Full Hexadecimal) Representation

2001:0db8:abcd:0000:0000:0000:0000:0000

## 3. Network Prefix (/64)

Network prefix (first 64 bits): 2001:0db8:abcd:0000  →  written as 2001:db8:abcd::/64

The remaining 64 bits (0000:0000:0000:0000) form the Interface Identifier (host portion), which is typically auto-generated (e.g., via SLAAC/EUI-64) for each connected medical device or IoT sensor.

## 4 & 5. Total Number of Host Addresses in the /64 Subnet

Host bits available = 128 − 64 = 64 bits.

Total possible addresses = 2^64 = 18,446,744,073,709,551,616 (≈ 1.8 × 10^19) addresses.

This single /64 subnet can therefore address roughly 18.4 quintillion devices — far more than enough for every current and future medical device, IoT sensor, or connected system in the hospital.

## Analysis

A /64 is the standard subnet size for IPv6 LANs specifically because it reserves the full 64-bit host portion for auto-configuration (SLAAC/EUI-64), giving the hospital effectively unlimited address space per subnet and removing any need for NAT or careful host-count planning as device numbers grow.

# Question 4: ISP Core Router Packet Forwarding

Routing table entries: 192.168.1.0/24, 192.168.2.0/24, 192.168.3.0/24. Destination IP of arriving packet: 192.168.2.45

## 1. Destination Subnet

192.168.2.45 falls within the 192.168.2.0/24 network (valid range 192.168.2.1 – 192.168.2.254).

## 2. Matching Routing Table Entry

192.168.2.0/24 — this is the longest/only prefix matching the destination address using the router's longest-prefix-match algorithm.

## 3. Next-Hop Network/Interface

The router forwards the packet out of the interface (or to the next-hop) configured for the 192.168.2.0/24 network — i.e., the interface directly connected to, or routing toward, that customer network. Since 192.168.2.0/24 has a direct routing entry, the router treats it as directly connected/reachable and delivers the packet to that interface for final delivery to host .45.

## 4. Default Route (No Match Case)

If the destination address had not matched any of the three specific entries, the router would forward the packet using its default route, typically expressed as 0.0.0.0/0, which points to a gateway of last resort (e.g., the ISP's upstream link) for onward delivery.

## Analysis

Routers always apply longest-prefix match; since 192.168.2.45 matches exactly one /24 entry, the decision is unambiguous. The default route (0.0.0.0/0) exists precisely to handle destinations — such as the wider Internet — that fall outside the three known customer subnets.

# Question 5: Two-Department LAN via Layer-3 Router

Administration LAN: 192.168.10.0/26  |  Finance LAN: 192.168.10.64/26

## 1 & 2. Network Address, Broadcast Address, and Valid Host Range

| LAN | Network Address | Broadcast Address | Valid Host Range |
| --- | --- | --- | --- |
| Administration | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 |
| Finance | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 |

## 3. Suggested IP Addresses for Three Computers per LAN

- Administration LAN: 192.168.10.2, 192.168.10.3, 192.168.10.4 (192.168.10.1 is reserved for the gateway)

- Finance LAN: 192.168.10.66, 192.168.10.67, 192.168.10.68 (192.168.10.65 is reserved for the gateway)

## 4. Default Gateway for Each LAN

- Administration LAN hosts → default gateway 192.168.10.1

- Finance LAN hosts → default gateway 192.168.10.65

## Analysis

Both /26 subnets are carved from the same 192.168.10.0/24 block with no overlap (0–63 and 64–127), each providing 62 usable hosts — comfortably enough for a typical department LAN — while the router's two interfaces (.1 and .65) act as the respective default gateways enabling inter-department routing at Layer 3.